Open-Ended Lab Report

Smart Lock Using ATMEGA

Course: Microprocessor Programming and Interfacing EL-3002

Instructor: Engr. Zohaib

| Name | Roll Number | Dept & Section |
| --- | --- | --- |
| Muzna Kamal | 23K-6002 | BSEE-A |
| Eshaal Ali | 23K-6075 | BSEE-A |
| Saman Rashid | 23K-6078 | BSEE-A |

Table of Contents

Introduction:	2

Motivation:	2

Problem Statement & Targeted SGGs:	2

Targeted SDGs:	2

Application Interface:	2

Proposed Solution:	2

Block Diagram:	2

Literature Review:	3

Methodology:	3

Step 1: Hardware Design	3

Step 2: Software Development	4

Step 3: Integration and Simulation	4

Code:	5

Costing:	9

Results:	9

Simulation:	9

Hardware Implementation:	10

Discussion:	11

Conclusion & Future Recommendations:	11

Conclusion:	11

Future Enhancement:	11

# Introduction:

## Motivation:

With increasing security concerns in homes, offices, and laboratories, traditional mechanical locks are no longer sufficient due to risks of key duplication and physical wear. Embedded-system-based electronic locks provide better security, flexibility, and user control. This project is motivated by the need to design a low-cost, reliable, and easy-to-implement digital door locking system using a keypad, LCD, and servo motor controlled by an ATmega32 microcontroller.

## Problem Statement & Targeted SGGs:

Unauthorized access to restricted areas leads to security breaches, theft, and safety risks. Many secure systems are either expensive or complex for small-scale deployment.

### Targeted SDGs:

#### SDG 9 – Industry, Innovation, and Infrastructure:

Promotes innovation through microcontroller interfacing and design.

#### SDG 11 – Sustainable Cities and Communities:

Enhances safety and access control in residential and institutional environments.

### Application Interface:

The system interfaces a 4×4 keypad for password input, a 16×2 LCD for user feedback, and a servo motor to physically lock/unlock the door, all controlled by the ATmega32 development board.

## Proposed Solution:

The proposed solution uses a password-based authentication mechanism. When the correct password is entered via keypad, the microcontroller validates it and rotates a servo motor to unlock the door. Status messages are displayed on the LCD.

### Block Diagram:

Figure 1: Architecture: Keypad is connected to PORTA, LCD is connected to PORT D, and Servo Motor to PORTC bit 0

Figure 2: Program Logical Flow

# Literature Review:

Several studies highlight the effectiveness of keypad and microcontroller-based locking systems:

- Password-based door lock systems using AVR microcontrollers.

- Embedded security systems using keypad and LCD interfaces.

- Servo motor-based access control mechanisms.

- Low-cost home automation and security solutions.

- Microcontroller-based electronic locks for smart homes.

- Comparison of biometric vs keypad security systems.

- Embedded systems for physical access control.

- Energy-efficient security systems using AVR.

- Design of digital locks using assembly language.

- Secure authentication techniques in embedded systems.

# Methodology:

## Step 1: Hardware Design

- ATmega32 microcontroller

- 4×4 matrix keypad connected to PORTA

- 16×2 LCD connected to PORTD (data) and PORTB (control)

- Servo motor connected to PORTC bit 0.

Figure 3: Hardware Design

## Step 2: Software Development

- Assembly Language Programmed using m32def.inc

- LCD initialization and command routines

- Keypad scanning using row-column technique

- Password storage in SRAM

- Servo motor control using PWM-like delay pulses

; Store keypad input in SRAM

ST X, R16

INC R22

CPI R22, 3

BREQ CHECK_PASS

Each key press is stored sequentially in SRAM. Once three characters are entered, the password verification routine is triggered.

## Step 3: Integration and Simulation

- Circuit designed and simulated in Proteus

- Verified correct password entry, LCD messages, and servo rotation

## Code:

# Costing:

| Component | Specification | Quantity | Unit Cost (PKR) | Total Cost (PKR) |
| --- | --- | --- | --- | --- |
| ATmega32 Microcontroller | 8-bit AVR MCU | 1 | 880 | 880 |
| 4×4 Matrix Keypad | Numeric Keypad | 1 | 150 | 150 |
| Servo Motor | SG90 / Equivalent | 1 | 200 | 200 |
| 16×2 LCD | Alphanumeric Display | 1 | 300 | 300 |
| Breadboard | Standard Size | 1 | 150 | 150 |
| AVR Programmer | USBasp Programmer | 1 | 580 | 580 |
| Total Project Cost |  |  |  | 2360 PKR |

# Results:

## Simulation:

Figure 4: Incorrect Password, door locked, motor OFF

Figure 5: Correct Password, Motor ON

## Hardware Implementation:

Figure 6: Password correct, door unlocked, Motor ON

Figure 7: Password Incorrect, Door Locked, Motor OFF

# Discussion:

The objective of secure access control is successfully achieved using a simple keypad-based authentication system. The system is reliable, fast, and easy to implement. Assembly-level programming provides full control over hardware timing, making the servo control precise. However, the system is limited to fixed passwords and does not include encryption or remote access.

# Conclusion & Future Recommendations:

## Conclusion:

A low-cost and efficient password-based door locking system was successfully designed and implemented using ATmega32. The system demonstrates practical application of embedded systems, interfacing techniques, and low-level programming.

## Future Enhancement:

- Add EEPROM-based password change feature

- Integrate RFID or biometric authentication

- Add GSM/Wi-Fi for remote monitoring

- Implement buzzer/alarm on multiple wrong attempts.